Assembly Project

Name: Rama Abdlrahman

Id:1191344

( I have used visUAL program to compile my code as Dr Abu Al Soud told us that we can use it )

the question:

In this Question , I need to implement a simple encryption and decryption algorithm in ARM assembly. Given a text message (string) stored in the memory, the encryption program gets the ASCII code of each character and apply this method *the method is: Invert the first and the second bits of the ASCII code of each character. The encrypted character is stored back in a different location in the memory.

I have to write two assembly programs; encryption and decryption. The encryption program takes the text message stored in the memory at specific address and applies the encryption method on every single character and then stores the encrypted message in a different location in the memory. The decryption program takes the encrypted message and applies the reverse of the encryption process, which is called the key.

the code:

;Rama Abdlrahman 1191344

;A program to  implement a simple encryption and decryption algorithm in ARM assembly. Given a text message (string) stored in the memory

;The encryption  gets the ASCII code of each character and  Invert the first and the second bits of the ASCII code of each character

;The	encrypted character is stored back in a different location in the memory

;The decryption program takes the encrypted message and applies the reverse of the encryption process, which is called the key.

MYSTRING			DCB		'R','a','m','a','A','B','D',0        ; allocates bytes of memory, and defines the initial runtime contents of the memory.

MSG1				FILL		8                     ; book 8 wordes in the memory

MSG2				FILL		8                      ; book 8 wordes in the memory

ENTRY

ADR      R0,MYSTRING            ; creat pointer R0 point at mystring

ADR	R3,MSG1                       ; creat pointer R3 point at msg1

ENCRYPTION_LOOP1

LDRB		R1,[R0]                  ; Load the value of R1 from the address R0

CMP		R1,#0                         ; compare  the value of r1 with zero

BEQ		DONE                          ;If the value of r1 is zero then jump out from the loop

EOR		R1,R1,#3                      ; Logical Exclusive OR two inverse the Two bits LSB of each char

STRB		R1,[R3]                       ; Store least byte from R1 to address held in R3

ADD		R0,R0,#1                      ; increment R0 by 1

ADD		R3,R3,#1                      ; increment R3 by 1

B		ENCRYPTION_LOOP1              ;Go back and do the loop

DONE

ADR		R0,MSG1                       ; creat pointer R0 point at MSG1

ADR		R3,MSG2                       ; creat pointer R3 point at MSG2

DECRYPTION_LOOP2

LDRB		R1,[R0]                       ;  Load the value of R1 from the address R0

CMP		R1,#0                         ; compare  the value of r1 with zero

BEQ		EXIT                          ; If the value of r1 is zero then jump out from the loop

EOR		R4,R1,#3                      ; Logical Exclusive OR two inverse the Two bits LSB of each char

STRB		R4,[R3]                       ; Store least byte from R4 to address held in R3

ADD		R0,R0,#1                      ; increment R0 by 1

ADD		R3,R3,#1                      ; increment R3 by 1

B		DECRYPTION_LOOP2              ;Go back and do the loop

EXIT

END

*************************************************************************

- How  the program work :

- my string in the program is "RamaABD"	:

the character --->	      ASCII CODE  ---> 	 BINARY     ---> Hexa

R                       --->	           082     ---> 	01010010   ---> 52

a                        --->	          097     ---> 	               01100001   ---> 61

m                       --->	          109     ---> 	              01101101   ---> 6D

a      	             --->                      097     ---> 	01100001   ---> 61

A	              --->                    065     ---> 	 01000001   ---> 41

B                            --->	          066     ---> 	                01000010   ---> 42

D                               --->	          068     --->               01000100   ---> 44

1- THE HEXA value of each char will be stored in the memory started from address 0x200 for example, so the first value 0x52  will be stored in address 0x200  . The second value 0x61  will be stored in address 0x201,........., the last value 0x44 for character D will be stored in 0x206, there will be no value in address 0x207

2-  the encryption loop will work in this way :

it will take the binary value  of each char and inverse the first and secod LSB using Logical Exclusive OR instruction:

the binary value of 'R' is 01010010 ----> after inverting :01010001 ---> the hexa value after encryption : 51 ,  it will be stored in address 0x208 , and so on for all char I have until we reach 0 (null) and jump out of the loop.

3-  the decryption loop will work in this way :

it will take the encrypted message and applies the reverse of the encryption process using Logical Exclusive OR instruction :

the binary value of the 'R'  after encryption is 1010001 ----> after inverting :1010010 ---> the hexa value after decryption : 52  ,  and	so on for all char I have until we reach 0 (null) and jump out of the loop.

******************************************************************

- Memory contents:

-these are the hexa values of the characters allocated in the memory :

0x52 is for R,    0x61 is for a,   0x6D is for m,    0x61 is for a,    0x41 is for A,

0x42 is for B   ,   0x44 is for D

-these are the hexa values of the characters allocated in the memory after inversing and  incryption  :

0x51 is for R,    0x62 is for a,   0x6E is for m,    0x62 is for a,    0x42 is for A,

0x41 is for B   ,   0x47 is for D

-these are the hexa values of the characters allocated in the memory after inversing and  decryption  : ( the same as the hexa values of the characters allocated in the memory at the begining of the program)

0x52 is for R,    0x61 is for a,   0x6D is for m,    0x61 is for a,    0x41 is for A,

0x42 is for B   ,   0x44 is for D

***************************************************************************

- The values of registers used(R0,R3,R4,R1) and the value of pc  and the values of CSPR status bits after excution:

- The symbols: