Signals and Systems Assignment

Student Name: Rama Abdlrahman

ID :1191344

Instructor: Dr. Ashraf Al-rimawi

Section:1

Q 1/ A

A. x (t)=π[(t -3)/A ]+π[(t -C) /B]

A = 3, B = 4 and C = 4.

A. x (t)=π[(t -3)/3 ]+π[(t -4) /4]

A = 3;

B=4;

C=4;

u = @(t) t >= 3 + 0.5 * A;

u1 = @(t) t >= 3 - 0.5 * A;

u2 = @(t) t >= C + 0.5 * B;

u3 = @(t) t >= C - 0.5 * B;

X = @(t) -u(t) + u1(t) - u2(t) + u3(t);

%to plots the signal

fplot(X)

xlim([-2.94 7.06])

ylim([0.13 2.13])

%to make  title for figure

title('X(t) = pi( ( t-3 ) /3 )+ pi( ( t-4) /4 ) ');

xlabel('t');

ylabel('X(t)');

*************************************************************

Q 1/ B

B.  xb (t)= r(t ) - r(t-A ) - r(t-B ) + r(t-C )

A = 3, B = 4 and C = 4.

xb(t)= r(t ) - r(t-3 ) - r(t-4 ) + r(t-4 )

syms Xb t;

y1=(t).*heaviside(t);

y2=(t-3).*heaviside(t-3);

y3=(t-4).*heaviside(t-4);

y4=(t-4).*heaviside(t-4);

Xb=y1-y2-y3+y4;

fplot(Xb);

xlabel('t');

ylabel('Xb(t)');

title (' xb(t)= r(t ) - r(t-3 ) - r(t-4 ) + r(t-4 ) ');

xlim([-10.0 12.5])

ylim([-1.62 5.13])

************************************************************************************************************************************************************************************************************************************************

Q2:

syms x1 x2 x3 m n t;

x1=sin(10.*pi.*t);

x2=(1/3)*sin(30.*pi.*t);

x3=(1/5)*sin(50.*pi.*t);

xb=x1+x2;

xc=x1+x2+x3;

%The period of signals, x1, xb and xc, is 1/5

% (5 is the frequency for x1)

subplot(2,2,1);

fplot(x1,[0 0.2]);

xlabel('t');

ylabel('x1(t)');

title ('x1=sin(10.*pi.*t)');

subplot(2,2,2);

fplot(xb,[0 0.2]);

xlabel('t');

ylabel('xb(t)');

title ('xb= x1+x2');

subplot(2,2,[3 4]);

fplot(xc,[0 0.2]);

xlabel('t');

ylabel('xc(t)');

title ('xc=x1+x2+x3');

t=0:.01:10;

y=10*sin(10*pi*t);

[~,idx]=findpeaks(y);

T=t(idx(2))-t(idx(1));

disp(['Time Period =' num2str(T),'seconds']);

- By changing the plotting time from 0.2 to a larger value (for example, 1), Matlab plots will give us the visual ouptput to show if the signal is periodic or not (signal repeating itself or not).

A signal is periodic if at some time t, it starts to repeat the same values, with time t*2 being another repeat and so on.

A sine wave with time 2*pi*F1 repeats itself F1 times per second. A sine wave with time 2*pi*F2 repeats itself F2 times per second. At time F1*F2 with F1 and F2 integer, we will have repeated the first signal due to F1, and the second signal due to F2. Therefore sin(2*pi*F1*t) + sin(2*pi*F2*t) will repeat itself no later than t = F1*F2 and must be periodic.

we know have a periodic signal based upon the sum of the first two signals, and we have a third signal that is another sine wave. By the same reasoning as before, there exists some time for which the composite signal and the third signal both repeat. Therefore the sum of the three signals is periodic.

As you see signal repeating itself :

************************************************************************************************************************************************************************************************************************************************

Q3:

sympref('HeavisideAtOrigin',1);

syms taw t;

A = 3; B = 4; C = 4;

x = (exp(-2*taw) - C*exp(-10*taw)).*heaviside(taw);

%input signal

h = rectangularPulse((t-B)-taw/A); %impulse response

conv= int (x*h, taw, -inf, inf )    %output signal

SimpleSolution = simplify(vpa(conv))

fplot( conv, [-2  12]);

xlabel("t");

ylabel("(x*h)(t)");

title("the signal y(t) which is the convolution of x(t)and h(t)");

conv =

heaviside(t - 7/2)*((2*exp(-30*t)*exp(105))/5 - (exp(-6*t)*exp(21))/2 + 1/10) - heaviside(t - 9/2)*((2*exp(-30*t)*exp(135))/5 - (exp(-6*t)*exp(27))/2 + 1/10)

SimpleSolution =

heaviside(1.0*t - 3.5)*(1.595807828218886340305502911492e+45*exp(-30.0*t) - 659407867.24160734860499944187265*exp(-6.0*t) + 0.1) - 1.0*heaviside(1.0*t - 4.5)*(1.7053559793258884179574746752306e+58*exp(-30.0*t) - 266024120300.89930834187365217059*exp(-6.0*t) + 0.1)

************************************************************************************************************************************************************************************************************************************************

Q4:

% Question 4 (A)

syms t y(t) %symbolic variable

A = 3; B = 4; C = 4;

ode = diff(y,t,2) + A*diff(y,t) + B*y(t) == C + 5*cos(1500*t); %given eqn

con1 = y(0) == 0; %y(0)

Dy(t) = diff(y,t); %y'(t)

con2 = Dy(0) ==0; %y'(0)

solution = dsolve(ode,[con1 con2]) %output y(t)

simpleSolution = simplify(solution) %simplified representation

fplot(solution,[0  20])

ylim([0  1.2]);

xlabel("t");ylabel("y(t)");title("The Output");

solution =

cos((7^(1/2)*t)/2)*(cos((7^(1/2)*t)/2) - (2812495*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 - (3*7^(1/2)*sin((7^(1/2)*t)/2))/7 - (8437501875*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 - (8437515*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 + sin((7^(1/2)*t)/2)*(sin((7^(1/2)*t)/2) + (5625*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (5625*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (2812495*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 + (3*7^(1/2)*cos((7^(1/2)*t)/2))/7 + (8437515*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) - (3796885125027*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528

simpleSolution =

(5625*sin(1500*t))/1265625562504 - (2812495*cos(1500*t))/1265625562504 - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 - (3796885125027*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528 + 1

******************************************************************************

Q4:

% Question 4 (B)

syms t y(t) %symbolic variable

A = 3; B = 4; C = 4;

ode = diff(y,t,2) + A*diff(y,t) + B*y(t) == C + 5*cos(1500*t); %given eqn

con1 = y(0) == 0; %y(0)

Dy(t) = diff(y,t); %y'(t)

con2 = Dy(0) ==A; %y'(0)

sol = dsolve(ode,[con1 con2]) %output y(t)

simpleSolution = simplify(sol) %simplified representation

fplot(sol,[0  20])

ylim([0  1.2]);

xlabel("t");ylabel("y(t)");title("The Output");

sol =

cos((7^(1/2)*t)/2)*(cos((7^(1/2)*t)/2) - (2812495*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 - (3*7^(1/2)*sin((7^(1/2)*t)/2))/7 - (8437501875*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 - (8437515*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 + sin((7^(1/2)*t)/2)*(sin((7^(1/2)*t)/2) + (5625*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (5625*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (2812495*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 + (3*7^(1/2)*cos((7^(1/2)*t)/2))/7 + (8437515*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) + (3796868249997*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528

simpleSolution =

(5625*sin(1500*t))/1265625562504 - (2812495*cos(1500*t))/1265625562504 - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 + (3796868249997*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528 + 1

********************************************************************************

Q4 A&B together:

% Question 4 (A)

syms t y(t) %symbolic variable

A = 3; B = 4; C = 4;

ode = diff(y,t,2) + A*diff(y,t) + B*y(t) == C + 5*cos(1500*t); %given eqn

con1 = y(0) == 0; %y(0)

Dy(t) = diff(y,t); %y'(t)

con2 = Dy(0) ==0; %y'(0)

solution = dsolve(ode,[con1 con2]) %output y(t)

simpleSolution = simplify(solution) %simplified representation

fplot(solution,[0  20], '-b')

hold on

% Question 4 (B)

syms t y(t) %symbolic variable

A = 3; B = 4; C = 4;

ode = diff(y,t,2) + A*diff(y,t) + B*y(t) == C + 5*cos(1500*t); %given eqn

con1 = y(0) == 0; %y(0)

Dy(t) = diff(y,t); %y'(t)

con2 = Dy(0) ==A; %y'(0)

sol = dsolve(ode,[con1 con2]) %output y(t)

simpleSolution = simplify(sol) %simplified representation

fplot(sol,[0  20], '--r')

ylim([0  1.2]);

xlabel("t");ylabel("y(t)");title("The Output");

legend('dy/dt(0)=0','dy/dt(0)=A' );

>> Q4theTwoPartsTogether

solution =

cos((7^(1/2)*t)/2)*(cos((7^(1/2)*t)/2) - (2812495*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 - (3*7^(1/2)*sin((7^(1/2)*t)/2))/7 - (8437501875*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 - (8437515*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 + sin((7^(1/2)*t)/2)*(sin((7^(1/2)*t)/2) + (5625*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (5625*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (2812495*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 + (3*7^(1/2)*cos((7^(1/2)*t)/2))/7 + (8437515*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) - (3796885125027*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528

simpleSolution =

(5625*sin(1500*t))/1265625562504 - (2812495*cos(1500*t))/1265625562504 - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 - (3796885125027*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528 + 1

sol =

cos((7^(1/2)*t)/2)*(cos((7^(1/2)*t)/2) - (2812495*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 + (5625*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 - (3*7^(1/2)*sin((7^(1/2)*t)/2))/7 - (8437501875*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 - (8437515*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 + sin((7^(1/2)*t)/2)*(sin((7^(1/2)*t)/2) + (5625*cos(1500*t - (7^(1/2)*t)/2))/2531251125008 - (5625*cos(1500*t + (7^(1/2)*t)/2))/2531251125008 + (2812495*sin(1500*t - (7^(1/2)*t)/2))/2531251125008 - (2812495*sin(1500*t + (7^(1/2)*t)/2))/2531251125008 + (3*7^(1/2)*cos((7^(1/2)*t)/2))/7 + (8437515*7^(1/2)*cos(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437515*7^(1/2)*cos(1500*t + (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t - (7^(1/2)*t)/2))/17718757875056 + (8437501875*7^(1/2)*sin(1500*t + (7^(1/2)*t)/2))/17718757875056) + (3796868249997*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528

simpleSolution =

(5625*sin(1500*t))/1265625562504 - (2812495*cos(1500*t))/1265625562504 - (1265622750009*exp(-(3*t)/2)*cos((7^(1/2)*t)/2))/1265625562504 + (3796868249997*7^(1/2)*exp(-(3*t)/2)*sin((7^(1/2)*t)/2))/8859378937528 + 1

*note: I have used the online Matlab .. and with difficulty I knew how to download files from it to my device, so if there is any defect in the  the matlab files and if any of them are missing..please tell me.. most likely there will be no problems, but I attached this note as a precaution..and thanks!